Kings Worthy Parish Council

Maintenance Schedules (page 1 - 4) and Summary Pricing Schedule (page 5)

The Contractor shall provide an overall cost for the maintenance of each Open Space in Kings Worthy as shown on the Summary Pricing Schedule on page 5. These will comprise operations itemised in the Maintenance Schedule below.

The Maintenance Schedule on pages 1 to 4 lists the Operations, Timing and Frequency to be repeated each year for duration of maintenance period.

The schedules state the timing of key annual operations for specific soft landscape types, as shown on the drawings.

For required standard, the schedules should be read in conjunction with Specification for Maintenance and BS7370:Part 4 Grounds maintenance - Recommendations for maintenance of soft landscape.

The contractor shall provide rates for completion of a single operation, for the items where spaces are provided.

N.B. Rates given may be used to omit items by the Supervising Officer as deemed appropriate based on prevailing weather and site operational requirements.

| Schedule 1: General (site wide) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Operation | Frequency | J | F | M | A |  | M | J | J | A | S | O | N | D | Rate per operation |
| 1 | Litter pick all areas | Monthly (weekly for hard surfaces) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Litter bin emptying | Weekly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Fly tipping | As required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Removal of fallen leaves and small branches | Three a year |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Hard surfaces – inspect and keep clean | Weekly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Weed control around fence posts, fencelines, bollards etc. | Monthly Apr-Sept |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Schedule 2: Trees |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Operation | Frequency | J | F | M | A | M |  | J | J | A | S | O | N | D | Rate per operation |
| 1 | Weed Control to all tree surrounds | Monthly from March to October |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Watering | Twice Monthly if required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Check tree support (adjust & replace as required) | Twice annually |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Pruning fruit trees (subject to species) | Once annually |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Mulching – topping up | Once annually |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Schedule 3: Hedgerows |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Operation | Frequency | J | F | M | A | M | J | J | A | S | O | N | D | Rate per operation |
| 1 | Cutting hedges and vegetation (e.g. dog walk path) within KWPC land | Varies, see specification |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Cutting hedges adjacent highways & footpaths | Twice annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Cutting formal hedges (Burial Ground, Lovedon Fields allotments & Fraser Road) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Schedule 4: Ornamental Shrub Areas |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Operation | Frequency | J | F | M | A | M | J | J | A | S | O | N | D | Rate per operation |
| 1 | Weeding | Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Pruning | Once annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Removal of self-set suckers and seedlings at commencement of contract | Once |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Schedule 5: Grass Areas |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Operation | Frequency | J | F | M | A | M | J | J | A | S | O | N | D | Rate per operation |
| 1 | Mowing amenity grass areas (various locations) | Fortnightly from March to October |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Strim around features (excl 50mm from) within amenity grass areas. | Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Mowing edge of path, orchard area and top of SUDS basin at Lovedon Fields | 4-weekly between April and October (inclusive) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Mowing wildflower areas (various locations) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Mowing 1/3 of scrub margins at Lovedon Fields |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Allow for removal of notifiable and invasive weeds (e.g. ragwort at Burial Ground & buddleia at Lovedon Fields) before seeds set. | At least once annually |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Schedule 6: Sports pitches |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Operation | Frequency | J | F | M | A | M | J | J | A | S | O | N | D | Rate per operation |
| 1 | Mowing football pitches to maintain specified height | As required |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Verti-draining football pitches | Twice annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Repairs during the season | As required |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Marking out/lining football pitches & cricket strip | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Re-marking football pitches & cricket strip | As required |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Letting down and taking up cricket nets | Once annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | End of season renovations – football pitches | As required |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Summer season remedial works – football pitches | As required |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | Pre-season maintenance works – football pitches | As required |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Schedule 7: Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Operation | Frequency | J | F | M | A | M | J | J | A | S | O | N | D | Rate per operation |
| 1 | Burial Ground – reinstatement of a burial plot | Item |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Burial Ground - Empty 2 compost bins | Item |  |  |  |  |  |  |  |  |  |  |  |  |  |

Kings Worthy Parish Council

Grounds Maintenance Contract 2026-27 - Summary Pricing Schedule

| Site | Annual cost |
| --- | --- |
| Church Green |  |
| Hinton Park including Hinton Fields areas (west & east) |  |
| Lovedon Fields |  |
| Eversley Park |  |
| Broadview (upper and lower) |  |
| Kim Bishop Walk |  |
| Fraser Road open space |  |
| Fryers Close |  |
| Lovedon Lane Burial Ground |  |
| Total |  |